Paper 1 — Design & Proposal

Canopy-Aware Earth-Observation Gaussian Splatting with Spaceborne-Lidar Supervision

Separating canopy from bare ground in vegetated terrain from optical satellites, with calibrated per-pixel height uncertainty.

Candidate: Navaneeth  ·  UAH Computer Science PhD  ·  NASA ESDS / Earth Observation

Prepared for advisor review  ·  June 2026  ·  Status: design — no training code until sign-off

Contents

# 1. Executive summary

Thesis. Fuse NASA spaceborne lidar (GEDI L2A/L2B and ICESat-2 ATL08) into a multi-date satellite 3D Gaussian Splatting reconstruction so that it correctly separates the canopy surface from the bare ground in vegetated terrain — the documented failure mode of every current EO-3DGS method — and emits a calibrated per-pixel height uncertainty. Deliverables: a canopy height model (CHM) and an under-canopy digital terrain model (DTM) from optical satellites, plus a usable confidence map.

Why now / why us. We have a faithfully reproduced EOGS baseline on the lab 4090 (JAX DSM MAE ≈ 1.44 m, matching the paper). Everything below is an addition to that working pipeline, not a rebuild. EO-specific 3DGS + spaceborne-lidar supervision is a NASA-native niche the CV groups racing on 3DGS cannot easily replicate, because it depends on GEDI/ICESat-2 access and EO domain context.

Contributions (three, escalating). (1) a lidar-anchored height loss supervising the rendered elevation at sparse GEDI/ICESat-2 footprints; (2) a two-surface canopy/ground decomposition letting a semi-transparent canopy layer sit over a ground layer, supervised by the two lidar returns; (3) a calibrated, exported per-pixel height uncertainty — under-explored in EO and the hook into the Paper 3 image-selection agent.

Ask of this review. Approve the niche, method, and evaluation plan; decide the four open questions in §9 so the build can start.

# 2. Novelty fact-check (current as of mid-2026)

The field moves monthly, so the niche was re-verified against current literature. Conclusion: the specific combination — spaceborne lidar fused into a satellite-RPC 3DGS for canopy/ground separation with calibrated uncertainty — remains unoccupied. Adjacent pieces exist separately and are cited, not redone.

## 2.1 What exists (cite, do not redo)

| Work | What it does | Why it is not us |
| --- | --- | --- |
| EOGS / EOGS++ (CVPR 2025 / 2511.16542) | RGB satellite 3DGS for DSM; our base method. | RGB-only surface; explicitly flags vegetation & low-coverage as failure modes. No lidar fusion, no canopy/ground split. |
| SkySplat (AAAI 2026) | Generalizable 3DGS from sparse multi-temporal satellite images. | RGB surface DSM; no spaceborne-lidar supervision, no vegetation structure. |
| ForestSplat (Remote Sensing 2025) | GS canopy-height maps, MAE ~0.17 m. | Close-range SfM/MVS drone imagery — not satellite, not spaceborne lidar. |
| Forest-digital-twin GS (RS 2025); TreeDGS (2026) | Forest GS from handheld/UAV laser + images; DBH from aerial GS. | Close-range / aerial rigs; ground-based lidar, not GEDI/ICESat-2, not satellite. |
| Under-Canopy Terrain Reconstruction (2601.22861) | Neural 3D + semantic canopy segmentation for under-canopy terrain. | Closest conceptual neighbor, but RGB ground-level imagery, no spaceborne-lidar fusion, no satellite/RPC setting. |
| DiffLiGS; LiDAR-enhanced 3DGS Mapping | Lidar-enhanced / diffusion-densified depth supervision for 3DGS. | Automotive/terrestrial lidar; not spaceborne, not satellite EO. |
| GEDI+ICESat-2(+Sentinel/SAR) canopy mapping (RS 17/12/1968; ECHOSAT) | 2D wall-to-wall canopy-height rasters, RMSE ~3.5–5 m. | 2D regression maps, not 3D reconstruction. We render view-consistent 3D CHM + under-canopy DTM. |
| Sat-NeRF (2203.08896), EO-NeRF | NeRF EO geometry; Sat-NeRF uncertainty down-weights transients. | NeRF (slow); uncertainty is internal-only, never exported/calibrated as a product. |

## 2.2 The open niche we claim

First method to:

- Fuse spaceborne lidar into a satellite RPC 3DGS — GEDI full-waveform (canopy top + ground) and ICESat-2 ATL08 (canopy & terrain photons) as a physical, NASA-native supervision signal, stronger than the stereo/monocular depth used by SparseSat-NeRF / Sat-DN (which is itself wrong under canopy).

- Separate canopy from bare ground in the documented EO-3DGS failure regime, producing CHM + under-canopy DTM from optical satellites — a real NASA product (biomass, carbon).

- Export a calibrated per-pixel height uncertainty on the DSM/CHM, rather than only using uncertainty internally to ignore moving cars.

# 3. Method foundation: where this grafts onto EOGS

We read the EOGS codebase (github.com/mezzelfo/EOGS, commit cca973e) to confirm the contributions attach to existing hooks rather than requiring a rewrite. Key facts:

- The renderer packs per-Gaussian colors_precomp = [rgb(3), altitude(1), constant(1)]; the CUDA rasterizer alpha-composites them. So it already renders a per-pixel elevation (altitude_render = render_pkg['render'][3]) and a coverage/opacity channel ([4]).

- The training loop already computes a (currently disabled) L_altitude_reference = (altitude_render - reference_altitude).abs() — a ready-made anchor point for lidar supervision.

- It already has an uncertainty term L_nll via gaussian_nll_loss with a per-pixel variance from transient_mask — currently used only to down-weight transients.

- Cameras expose ECEF_to_UVA / UVA_to_ECEF, UV_grid, reference_altitude, altitude_bounds — everything needed to project lidar footprints into a view and read the rendered altitude.

- Optimization params follow a clean w_L_<name> / iterstart_L_<name> convention, so new losses slot in with a weight and a start iteration.

# 4. Method — three contributions with concrete injection points

## 4.1 Contribution 1 — Lidar-anchored height loss

Idea. Project each GEDI footprint / ICESat-2 ATL08 segment into the current training view, sample the rendered altitude at those pixels, and penalize it against the lidar canopy-top and ground elevations. Because lidar is sparse, treat it as a strong but sparse anchor that the existing TV / view-consistency regularizers propagate spatially.

Where it goes.

| File | Change |
| --- | --- |
| scene/dataset_readers.py, scene/cameras.py | Load per-scene lidar anchors (footprint lon/lat → UVA pixel + canopy-top & ground altitude) alongside the existing reference_altitude. |
| train.py (loss block) | Sample altitude_render at anchor pixels; add L_lidar_canopy and L_lidar_ground terms beside the existing L_altitude_reference. |
| arguments/__init__.py (OptimizationParams) | Add w_L_lidar, iterstart_L_lidar following the existing convention. |

## 4.2 Contribution 2 — Two-surface canopy/ground decomposition

Idea. EOGS renders a single alpha-composited altitude (≈ first opaque surface, i.e. canopy top). Add a per-Gaussian vegetation/transmittance attribute so a semi-transparent canopy layer sits above a ground layer, and render a second 'ground' altitude channel. Supervise canopy-top vs ground with GEDI's two returns and ICESat-2's canopy vs terrain photons.

Two implementation routes (advisor decision — §9):

- CUDA route (cleanest). Extend diff-gaussian-rasterization to emit a transmittance-weighted last-surface altitude channel. More work (touches the CUDA kernel we built) but exact and efficient.

- Python prototype route (faster). Approximate the ground surface with a second render pass using re-weighted opacities (canopy Gaussians suppressed) / depth-peeling. Quick to validate the science before committing to CUDA.

Where it goes: scene/gaussian_model.py (new per-Gaussian attribute), gaussian_renderer/__init__.py (extend colors_precomp / second channel), plus the canopy/ground loss in train.py.

## 4.3 Contribution 3 — Calibrated, exported height uncertainty

Idea. Repurpose the existing L_nll / variance machinery to produce a calibrated per-pixel height uncertainty on the final DSM/CHM, not just an internal transient weight. Add a calibration step (temperature / variance scaling on held-out anchors) and export an uncertainty raster.

Where it goes: train.py (extend NLL to the height head), render.py (export uncertainty raster), plus a calibration utility. Evaluated with Expected Calibration Error (ECE) and a reliability diagram. This output is the intended consumer for Paper 3's uncertainty-driven image-selection agent.

# 5. Data & study areas (all open; feasibility checked)

Strategy: keep the established benchmark, add NASA-native supervision. Stay on the DFC2019 (US3D) tiles EOGS already uses so reviewers can compare directly; focus on vegetated tiles for the canopy story; keep built-up tiles to show no regression. GEDI/ICESat-2 provide sparse supervision; airborne lidar provides dense evaluation truth.

| Dataset | Role | Feasibility / notes |
| --- | --- | --- |
| DFC2019 US3D — JAX (Jacksonville FL), OMA (Omaha NE) | Multi-date WorldView-3 + airborne-lidar DSM truth (the EOGS benchmark). | Already on the 4090 (7 scenes). JAX is subtropical / vegetated — good canopy test. |
| GEDI L2A (RH + ground elevation), L2B (canopy cover, PAI) | Sparse canopy-top + ground supervision. | GEDI covers ±51.6° latitude → JAX (30.3°N) and OMA (41.3°N) both covered. A test earthaccess query already returned a GEDI granule over JAX. |
| ICESat-2 ATL08 (canopy & terrain heights) | Dense along-track canopy/terrain anchors. | Global coverage; 100 m along-track segments densify the sparse GEDI sampling. |
| USGS 3DEP airborne lidar (DTM; derive CHM) | Dense ground-truth for CHM/DTM evaluation. | US-wide incl. FL & NE; QL1/QL2 where available. |

## 5.1 Known data caveats (called out up front)

- Lidar is sparse per tile. GEDI footprints are ~25 m wide with ~600 m across-track beam spacing, so a ~600×600 m tile holds only a handful per overpass. Mitigation: aggregate GEDI over 2019–2024, add ICESat-2 ATL08, treat lidar as anchors that propagate — the sparse-anchor regime, not dense depth.

- Vertical datum alignment. GEDI/ICESat-2 elevations (ellipsoid/geoid) must be aligned to the scene's UTM altitude via the camera UVA transforms + pyproj. A real plumbing step, scheduled in M6.

- Site sufficiency. If the DFC2019 tiles prove too lightly vegetated, add one dedicated forested AOI with 3DEP + GEDI overlap and open WorldView/Maxar imagery (advisor decision, §9).

Next concrete step: a small scripts/10_query_lidar.py (earthaccess) to count GEDI/ICESat-2 footprints inside each tile, run on the 4090 where the Earthdata login is configured. Quantifies supervision density before any modeling.

# 6. Evaluation protocol & baselines (locked early)

Baselines (same tiles, fixed splits & seeds, released configs): EOGS / EOGS++ (RGB, direct rival), Sat-NeRF, EO-NeRF (cited), classical satellite MVS (S2P).

| Metric | What it measures | Where it applies |
| --- | --- | --- |
| DSM MAE (m) + completeness | Field-standard surface accuracy; must not regress on built-up. | All tiles (baseline parity) |
| Canopy-height MAE vs airborne CHM | Canopy-top accuracy. | Forested tiles |
| Under-canopy / DTM MAE | Bare-earth accuracy beneath canopy — the new capability. | Forested tiles |
| Uncertainty calibration (ECE, reliability diagram) | Whether the exported confidence is trustworthy. | All tiles |
| Train time | The EOGS speed story is part of the contribution. | All runs |

# 7. Ablation plan (the paper's evidence)

Show monotone gains on vegetated tiles and no regression on built-up tiles:

- RGB-only EOGS baseline (reproduced — done).

- + ICESat-2 ATL08 supervision.

- + GEDI supervision.

- + both lidar sources.

- + two-surface canopy/ground decomposition.

- + calibrated uncertainty head.

Stress test: reduce the number of views to probe the sparse-view regime (where we cite SkySplat rather than compete).

# 8. Risks & mitigations

| Risk | Mitigation |
| --- | --- |
| Lidar too sparse per tile to supervise effectively | Multi-year GEDI aggregation + ICESat-2; anchors-plus-propagation; quantify density with the query script before modeling. |
| Vertical-datum / geolocation misalignment | Careful ellipsoid/geoid → UTM handling via camera UVA transforms + pyproj; sanity-check vs airborne DSM. |
| Lidar fusion must actually beat RGB-only EOGS | The core empirical claim; the ablation ladder isolates each gain. Pick sites with genuine canopy + dense airborne truth. |
| Scooping (EO-3DGS moves monthly) | Move fast, arXiv early, lean on the NASA-native GEDI/ICESat-2 angle CV groups can't easily replicate. |
| Two-surface CUDA work balloons | Prototype the science in Python first (§4.2 route 2); commit to CUDA only once the gain is shown. |

# 9. Decisions needed from advisor (sign-off)

- Study sites: stay on DFC2019 JAX/OMA vegetated tiles only, or add one dedicated forested AOI (3DEP + GEDI overlap + open imagery)?

- Two-surface route: prototype in Python first, or invest in the CUDA rasterizer extension up front?

- Uncertainty scope: full calibration story in Paper 1, or a minimal exported uncertainty with calibration deferred?

- Venue / deadline: IGARSS (fast feedback), Remote Sensing (MDPI, fast OA), or aim higher at ISPRS J. Photogrammetry & RS?

# 10. Timeline (aggressive, ~3–4 months on the 4090)

| Milestone | Work | Est. |
| --- | --- | --- |
| M6 — Data & anchors | earthaccess pulls; GEDI/ICESat-2 → UVA anchors; datum alignment; density report. | 2–3 wk |
| M7 — Lidar loss | Contribution 1; ablations (i)–(iv) on vegetated tiles. | 2 wk |
| M8 — Two-surface | Contribution 2 (Python prototype → CUDA if warranted); CHM + DTM eval. | 3–4 wk |
| M9 — Uncertainty | Contribution 3 + calibration (ECE / reliability). | 2 wk |
| M10 — Sweep & write | Full ablation table, baselines, figures, draft + arXiv. | 3–4 wk |

Gate, as before: no new training code until this design is approved; each contribution is validated as an addition to the reproduced EOGS baseline.

# 11. Key references

- EOGS — Gaussian Splatting for Efficient Satellite Image Photogrammetry (CVPR 2025)  https://arxiv.org/abs/2412.13047

- EOGS++ — Internal Camera Refinement and Direct Panchromatic Rendering  https://arxiv.org/abs/2511.16542

- SkySplat — Generalizable 3DGS from Multi-Temporal Sparse Satellite Images (AAAI 2026)  https://arxiv.org/html/2508.09479

- ForestSplat — Scalable High-Fidelity Forestry Mapping with 3DGS (Remote Sensing 2025)  https://www.mdpi.com/2072-4292/17/6/993

- LiDAR-Guided Semantic 3DGS for Forest Digital Twins (Remote Sensing 2025)  https://doi.org/10.3390/rs18111696

- Under-Canopy Terrain Reconstruction Using RGB Imaging and Neural 3D Reconstruction (2026)  https://arxiv.org/abs/2601.22861

- Sat-NeRF — Learning Multi-View Satellite Photogrammetry (2022)  https://arxiv.org/abs/2203.08896

- Tropical Forest Canopy Height by Fusion of Sentinel-1/2 and Bias-Corrected ICESat-2–GEDI (RS 2025)  https://www.mdpi.com/2072-4292/17/12/1968

- GEDI Mission Overview (UMD)  https://gedi.umd.edu/mission/mission-overview/